# Smart Gate and Device Automation System

## Overview

This project implements a smart automation system using an Arduino Uno. An IR sensor detects objects and a servo motor opens the gate automatically. A relay module controls electrical devices, while an HC-05 Bluetooth module enables manual control from a smartphone.

## Features

- Automatic object detection using IR sensor

- Gate control using servo motor

- Device control using relay module

- Manual control via Bluetooth

## Hardware Components

| Component | Cost |

|-----------|----------------|

| Arduino Uno | ₹300–₹350 |

| IR Sensor | ₹25–₹50 |

| Servo Motor | ₹80–₹200 |

| Relay Module | ₹25–₹50 |

| HC-05 Bluetooth | ₹150–₹250 |

| Breadboard & Wires | ₹100–₹250 |

## Software

- Arduino IDE

- Bluetooth terminal app (Android)

## Working

1. IR sensor detects an object near the gate.

2. Arduino processes the signal.

3. Servo motor opens the gate automatically.

4. Relay switches a connected device.

5. Bluetooth allows manual control from a phone.

## Circuit Connections

**IR Sensor:** VCC→5V, GND→GND, OUT→Pin 2

**Servo Motor:** VCC→5V, GND→GND, Signal→Pin 9

**Relay Module:** VCC→5V, GND→GND, IN→Pin 7

**HC-05:** VCC→5V, GND→GND, TX→RX, RX→TX

## Applications

- Smart parking gates

- Home automation

- Automatic door systems

- Security access control

## Future Scope

- IoT control using ESP8266/ESP32

- Mobile application interface

- Additional sensors for security

## Author

Sneha

Electronics and Communication Engineering Student